BLACK SOAP MAKING PROCESS

EQUIPMENT FOR MAKING BLACK SOAP:

- Woven basket

- Mortar and pestle

- Drum or big pot

- Big frying pan

- Sack

- Turning stick

- Cup

- Small paint bucket

- Big bowl

- Calabash

- Incinerator

- Gas burner/ local Fire place

HOW TO MAKE THE CHEMICAL USED FOR MAKING BLACK SOAP (OMI-ARO):

- Collect dry cocoa pods and burn to ash in the incinerator.

- Collect the ash.

- Put a sack in the woven basket.

- Pour the ash in the sack in the woven basket.

- Put the basket on a stand.

- Put a bowl under to collect the filtrate.

- Keep pouring water in the ash until the filtrate is tasteless(the filtrate has a sharp taste).

- Keep filtrate for at least 2 days before cooking.

- Cook the filtrate until there is film formation on the surface of the water and presence of sandy substance at the base.

- Allow to cool down and store in a big keg.

HOW TO MAKE A BLACK SOAP

MATERIALS NEEDED:

- PKC (palm kernel cake)

- PKO (palm kernel oil)

- Omi-aro(chemical).

PROCEDURE:

- Pour a specific measurement of PKO in a pot or drum on fire (15lites)

- Leave the oil to boil.

- Measure a specific amount of omi-aro in a separate container (12litres).

- Measure out 1litre from the already measured omi-aro, add it to the boiled oil on fire.

- Stir thoroughly.

- Allow to boil until there are whitish bubbles coming out.

- Add about 4litres of omi-aro and stir properly.

- Allow to boil for some minutes, till there are whitish bubbles on the surface.

- Add about 3litres of omi-aro and stir properly.

- Allow to boil for some minutes till there whitish bubbles.

- Measure 5 small paint rubber of PKC, add it to the soap on fire.

- Stir it thoroughly and allow to cook.

- Continually add 1litre of omi-aro at interval till the soap is completely done (the soap brings out whitish bubble on boiling).

NOTE: Ensure you don't add excess omi-aro because it may cause damage to the soap.

- Pound the soap in a mortar.

- Spread a sack on the floor, place the pound soap on the sack, wrap with the sack to form a cone shape.

- Put a frying pan on fire.

- Put the soap in the frying pan with its base in the frying pan.

- Allow the soap to melt out.

- Pack using a calabash, spread on a sack to cool down.

- Pack and store in a container.

- The black soaps are ready for packaging and sale.